Basler Cable USB 3.0, Micro B sl/A, S#

USB 3.0 cable for data transmission with type A plug to Micro-B plug.

Info

The cable is suitable for ace and ace 2 cameras.

| Order Number | Cable Length |
| --- | --- |
| 2200000620 | 1 m |
| 2200000621 | 3 m |

Cable Drawing#

Transfer Rates#

|  | 2200000620 / 2200000621 |
| --- | --- |
| USB 2.0 | 480 Mbps |
| USB 3.0 | 5 Gbps |

Physical Specifications#

|  | 2200000620 / 2200000621 |
| --- | --- |
| Camera-Side Connector | Micro-B with screw lock |
| Host-Side Connector | Type A, USB-IF-compliant |
| Cable Diameter | 5.7 mm max. |
| Minimum Bending Radius | 28 mm, fixed installation |
| Maximum Bending Cycles | None (fixed installation only) |
| Suitable for Drag Chain Applications | No |
| Suitable for Robotics Applications | No |